- FACULTY OF ENGINEERING AND APPLIED SCIENCE

- ENGR3200U – Engineering Graphics and Design

- Unigraphics NX5 Learning Module # 2

ADVANCED BASICS

- LEARNING MODULE 2:

- ADVANCED BASICS

Learning Objectives:

| - Create 2D sketches using: Rectangles, Circles - Create a 3D object from sketches - Use the EXTRUDE function - Use the addition and SUBTRACTion Boolean operations | - Create inferred dimensions - Use the MATE function and various mates - Use the ASSEMBLY function |
| --- | --- |

ADVANCED BASICS – CABINET DESIGN

STEP 1:	Open up Unigraphics and create a new part file named Cabinet Shell (or similar name).  Refer to Steps 1 thru 3 of Learning Module 1 for creating a new file and ensuring that you are in Modeling mode.

STEP 2:	Click SKETCH to open the Create Sketch menu.  Choose On Plane for Type, select Existing Plane for Plane Option, and select the X-Y plane (you can also create a new datum plane on the X-Y axis by selecting Create Plane in Plane Option).  Click OK.

Click Rectangle (), which will then open the menu on the right.  From here, select Create from Centre ().  Create a rectangle 1500 mm x 600 mm with its centre at (0, 0).  Once you have drawn the rectangle, click Finish Sketch to exit the sketch mode.  Refer to the figure below for further dimensioning information.

STEP 3:	To make a solid from the sketch created, select EXTRUDE (), click on the sketch so that Unigraphics knows what profile you would like to extrude, and set the extrusion distance to 400 mm (end value).  Then click Apply and close the extrusion menu window.  See figure below for appropriate extrusion settings.

STEP 4:	To hollow out the rectangular solid, you will use the SHELL command.  Select the SHELL function (); now select the face of the rectangular solid to remove material from that side.  Under Type, select “Remove Faces, Then Shell”.  Enter a thickness of 50 mm and click Apply.  Then close the Shell window.

(The SHELL function also offers a shelling of an object so that each side can be given a different thickness (under Alternate Thickness, you select the face to give a different thickness, and enter the value of that thickness)).

STEP 5:	Now you will create a new sketch and extrude-cut that to create holes where the hinges will go.  Click on SKETCH again.  The sketch Type should be On Plane, and Plane Option should be Existing Plane.  Click OK.

STEP 6:	Now that you have your datum plane, create the holes necessary to attach the hinges to the cabinets and cabinet doors.  Create four 5 mm diameter circles with the following inferred dimensions.  Once all the circles are in their proper places (should have four circles), then click Finish Sketch.

STEP 7:	From here you will create the holes.  Select EXTRUDE, click on the circles you just drew for the holes, and extrude the circles to create the holes.  Under Limits, set Distance to 0 for Start value and -50 mm for End value.  In the Boolean menu located within the extrusion window, select SUBTRACT ().  Doing so will highlight the entire cabinet body, which means material will be subtracted from that body.  Select Apply.  If the cabinet body is not selected then click on Select Body option under the Boolean tab, and click on the cabinet body to select it.

(The diagram on the right is the Boolean pull-down menu located in the extrusion window, giving you the option to create, unite, SUBTRACT, or intersect the extrusion to be created with whichever solid the user desires).

STEP 8:	You also need to create the same holes on the other side of the cabinet.  This can be easily done by changing the extrusion parameters.  Using the EXTRUDE function again, set the start value as -550 mm and the end value to be -600 mm since you know the cabinet is 600 mm wide and you want to create 50 mm deep holes.

(Note that you can actually save yourself a couple of steps in creating the holes on both sides of the cabinet by setting Start and End to Through All and selecting the SUBTRACT Boolean operation.  This way, the material is removed at the same time from both sides of the cabinet).

STEP 9:  	Now you will create holes for the shelf supports, so that shelves can be inserted at a later time.  Using the previous datum plane used for the hinge holes, create 2 circles each with a 5 mm diameter located as depicted below.  Once you have completed this, click Finish Sketch.

STEP 10:	Click EXTRUDE, select the new drawing of the shelf holes, and select SUBTRACT under Boolean operations in the extrusion window.  Set the start value as -25 mm and the end value as -575 mm and SUBTRACT that portion.  Notice here that these holes do not go all the way through.

In this method you take down two birds with one stone.  Instead of having to redraw the circles on the other side and making a small cut-extrusion, you just make 1 long cut-extrusion that reaches both sides.

STEP 11:	Now you will use a new feature – INSTANCE FEATURE.  It is available under EXTRACT (see figure on right) or under Insert>Associative Copy.  Here, you will create a rectangular pattern.  You will see the Instance menu (lower right).  You can also customize your toolbar to have quicker access to this function.  Select Rectangular Array.

(The INSTANCE FEATURE function contains three options such as the Rectangular Array, Circular Array, and Pattern Face.  Arrays are predefined patterns which can be altered to fit most specifications.  Pattern Face is used for more complex details which allow the users to specify what they want to pattern, specify the X and Y directions, and define the borders of where the pattern should stop).

Once you select Rectangular Array, you should see the menu on the left.  This menu lets you select which features to be patterned.  Select the feature of the two cabinet shelf holes that were created in Step 10 (they will be highlighted in orange when you select the correct feature from the menu).  Click OK.  The menu on the right will appear.  You will create 13 instances of the shelf holes.  Set the parameters as follows:  Number Along XC = 1, XC Offset = 1, Number Along YC = 13, YC Offset = 100.

(Number Along XC/YC is the number of instances of the pattern you want along the X axis and Y axis.  XC/YC Offset is the distance desired between each instance to be patterned).

Once you select OK, you should end up with something similar to the picture below.  If this is what you see, select Yes to create the pattern, and close INSTANCE FEATURE.

This concludes the making of the Cabinet Shell.  Your final product for this object should look similar to the image below.

Now you will create the cabinet doors.  Save the cabinet shell; it will be used later on in this lab to create a fully functional cabinet.

ADVANCED BASICS – CABINET DOORS

STEP 12:	Now you will begin creating the Cabinet Doors.  Start up a new file, name it however you see fit (preferably something like Left Cabinet Door or something similar).

STEP 13:	Create a rectangular sketch on the XY plane 1500 mm x 300 mm as depicted on the right.  Then select Finish Sketch.

STEP 14:	Once the sketch is done, select EXTRUDE and set the starting value to be 0 and the ending value to be 50 mm as depicted below.

STEP 15:	Here you will make matching hinge holes that will line up with the cabinet’s hinge holes so that the cabinet doors can be installed properly.  Create a datum plane on one of the long thin sides of the cabinet door.  If you want this to be the left door create the datum plane on the left thin side, if you want this to be the right door, create the datum plane on the right thin side.

STEP 16:	Create a sketch of 4 circles with a diameter of 5 mm each (along the centre line of the side).  The location of these holes is depicted in the picture below with inferred dimensions you can use.  Once you have sketched the circles, select Finish Sketch.

STEP 16B: Save and then Save as a copy and rename it to door_right.

STEP 16C: Open the left door and follow the next step.

STEP 17:	Click EXTRUDE and select the start value to be 0 and the end value to be 50 mm (or -50 mm depending on from where you start; either way you want the extrusion to cut into the cabinet door).  Select SUBTRACT as the Boolean operation in the extrude menu.  Select Apply and close the extrusion window.

Once you have completed the extrusion cut, save the file, making sure to include whether it is the left or right door that you have just created, in the filename.

STEP 17B:	Click Save, and Open the right door part.

STEP 18:	Similar to Step 17, change the extrusion parameters on the part copy that will be the right door to extrude cut holes on the other end.  Click EXTRUDE and select the holes.  Select the SUBTRACT Boolean operation, and set the start value to be -250 mm and the end value to be -300 mm.  Click Apply.

At this point, you have created the cabinet shell and the left and right cabinet doors.  Now you will create the hinge set and hinge pin so that you can attach the doors to the cabinet itself.

ADVANCED BASICS – HINGE SET

STEP 19:	Just as you did with all solids objects that you made in Unigraphics, you must start again with a simple sketch.  Name the file Hinge A (or similar; Hinge B, or similarly named, will be the matching half to Hinge A).  Create a rectangle on the XY Plane with dimensions 50 mm x 70 mm.

STEP 20:	Make the sketch into a solid by extruding it.  Select EXTRUDE and set the start value as 0 and the end value as 2 mm.  Click Apply and close the extrusion window.

STEP 21:	Now you need to create the point at which the hinge will swivel upon.  Create a sketch on the ZX plane, drawing a circle with a diameter of 10 mm.  It should be drawn as depicted below.  Once done, select Finish Sketch.

(As can be seen, the centre of the circle is at the point (-25, 0, 5) so that it just barely touches the bottom of the hinge base and is perfectly centred so that half the circle will be made over the base while the other half does not touch it).

STEP 22:	Select EXTRUDE and select the circle sketch.  For this particular case you want the extrusion to be going across the entire base which is 70 mm in length.  Select Symmetric Value in the Start limit drop-down menu (automatically, Symmetric Value is selected for End limit) and input 35 mm for its value.  The end value will also automatically change to 35 mm.  You also want to combine this extruded cylinder with the base, so select UNITE for the Boolean operation.  Click Apply.

STEP 23:	Create a datum plane on the end of the object using the Existing Plane option, and selecting the surface shown in the figure below.

STEP 24:	Create a sketch of a circle at the centre point of the circular part of object with a diameter of 5 mm.  Select Finish Sketch once completed.

STEP 25:	Extrude the 5 mm diameter circle through the entire cylindrical part.  Select Through All for the starting and ending limits.  Select SUBTRACT under Boolean operations and click Apply, and then close the extrusion window.  Refer to the figure below for extrusion parameter settings.

STEP 26:	Now you will create two holes for mounting the hinge to the cabinet (via fasteners/screws that you will create later).  Create a sketch in the XY plane, and place two circular sketches according to the dimensions shown below.  The two holes are 5 mm in diameter and 30 mm apart.  Click Finish Sketch.

STEP 27:	Extrude the two circular sketches and perform the Boolean SUBTRACT operation with these two circles.  You may have to rotate your model to see the circular sketches, or you can select them directly from the Model History in the Part Navigator.  Set the start and end limits to Through All.  Click Apply and close the extrusion window.

STEP 27 B: Once you have accomplished the extrude cut, save the part file, and then use “Save As…” (under the File menu) to save a copy of the file.  This second copy will be used to create the matching half of the hinge set.

STEP 28:	Reopen the Hinge A part file.

STEP 28B: Create a sketch on the XY plane.  You want to draw two rectangles of dimensions 10 mm x 14 mm.  First a 14 mm gap, then the rectangle, then another 14 mm gap, then the other rectangle, and finally another 14 mm gap.  All required dimensions are depicted below.

STEP 29:	Unite the extruded rectangle and cylinder together before subtracting the two rectangles.

STEP 29 B:	Extrude the two rectangular sketches and perform the Boolean SUBTRACT operation with these two rectangles.  You may have to rotate your model to see the rectangular sketches, or select them directly from the Model History in the Part Navigator.  Set the start and end limits to Through All.  Click Apply and close the extrusion window.  Save the part file.

STEP 30:	Open the file copy (Hinge B or similarly named) you created before proceeding with Steps 28 and 29.  This time, you will create three rectangles instead of two.  Choose the XY plane as your data plane, and draw the three rectangles according to the dimensions given below (again, each is 10 mm x 14 mm).  Click Finish Sketch when done.

STEP 31:	Unite the extruded rectangle and cylinder together before subtracting the two rectangles.

STEP 31B: Extrude the two rectangular sketches and perform the Boolean SUBTRACT operation with these three rectangles.  Again, you may have to rotate your model to see the rectangular sketches to select them, or select them directly from the Model History in the Part Navigator.  Set the start and end limits to Through All.  Click Apply and close the extrusion window.  Save the part file.

You should have two hinge components that look like the figures below.

Now you have completed the cabinet shell, cabinet doors, and the hinge set.  The last thing you need to do is create the hinge pin that will hold the two hinge components together, as well as the fasteners to hold the hinges to the cabinet, and then you can start putting it all together.

ADVANCED BASICS – HINGE PIN

STEP 32:	Create a new part file; name it Hinge Pin (or something similar).  Create a circle on the XY plane with a diameter of 5 mm.

STEP 33:	Extrude the circle sketch 70 mm.

STEP 34:	Create a datum plane on top of the extruded object and draw a sketch of circle 10 mm in diameter.  Then extrude the sketch by 2 mm and unite the two objects.  Save the file once you have accomplished this.

You can also modify this pin to create the fasteners that will hold the hinges to the cabinet (we will not worry about complexities such as threading).  Simply right-click on the 70 mm extruded portion directly or through the Model History and select Edit Parameters.  Then change the extrusion end value to 50 mm (since the holes on the side of the cabinet are 50 mm deep; there will also be an extra 2 mm depth available due to the hinge thickness).  Once you have modified the part, save it under a new name (i.e., Hinge Pin II).  Now that all the necessary parts are created, it is time to put them all together.

ADVANCED BASICS – ASSEMBLY

STEP 35:	Now you have come to the point where you will begin assembling different parts together.  Create a new file – call it Full Assembly (or something similar).  By default, the Assembly Toolbar will appear at the bottom (this is true anytime you open a new or existing part file).

If you do not see this toolbar, go to Start (), and select Assemblies from the drop-down menu.  If you still do not see the toolbar, click any of the empty grey area outside the modeling window and select Assemblies (see figure on left) from the menu that appears.

Select ADD COMPONENT ().  From there you should see the Add Component window, as shown in the figure below.  Click Open, and select Cabinet Shell.  Click OK.  The part will appear in the Loaded Parts list and a Component Preview window will also appear showing the loaded part.  Other parts may also be listed if you have recently worked with them when you first open the Add Component window.

(Under Placement, you have four options for Positioning:  Absolute Origin, Select Origin, By Constraints, and Reposition.  Absolute Origin places the object as is, seen in the Component Preview at its original location.  Select Origin allows you to change the position of the object by moving its origin, but the original orientation will remain the same.  You will cover Mate in the next few steps.  Repositioning is used when the position/orientation of the object in component preview is undesirable, and works similar to Select Origin, but you can also rotate and translate the object to a new position after initial placement.  For example, maybe you want the cabinet to be upright instead of lying down.  For now, use Absolute Origin.

Under Settings, you have both Reference Set and Layer Option.  Reference Set should be Model.  Layer Option lets the user choose whether to leave the object as is (Original), change something about the object (Work), or specify certain conditions and constraints.  By default, it will be set to Original).

At this point, accept the default parameters and click OK.  The Cabinet shell will appear in the modeling window, placed at its origin.  If you were to select Reposition or Specify Origin for placement, you will see a window at first that will allow you to specify additional parameters for positioning the object.

STEP 36:	Now that you know how to import objects, you need to import the remaining pieces.  Follow Step 35 but with a few changes.  You will now add Hinge A to the assembly, so select Add Component again, go click Open, and select Hinge A (or however you may have named it), accept parameters and settings as before, and click Apply.  You will see the Assembly constraints window shown on the right.  The Mating Conditions window allows users to create different types of constraints as listed under Type You will go through most of the functions in this window because they are all very important when creating assemblies.

Constraint Types Explanation

Touch - Mate two faces/edges together.

Align - Mate two faces/edges so they are aligned with each other.

Angle - Mate a face/edge to be on a specific angle to another face/edge.

Parallel - Mate two faces/edges to be parallel to each other.

Perpendicular - Mate a face/edge to be perpendicular to another face/edge.

Center - Mate the centre of a face/edge to be centre with another face/edge.

At a Distance - Mate a face/edge to be a specific distance away from another face/edge.

Concentric - Mate a face/edge to be concentric to a round face/edge.

(NOTE:  Hinge 1 in the component preview may need to be rotated to see the proper faces which need to be mated.)

Touch () one face of the Cabinet Shell and the outside face of Hinge A as shown in the picture below.

(Select , and in the Component Preview select the outside face of Hinge A.  Then, in the Modeling Window with the Cabinet Shell, select a side face.  Keep the mating conditions window opened.)

STEP 37:	In the Assembly Constraints window, select the Center concentric constraint ().  Then select the rear edge of one of the holes in the hinge and then select the front edge of the corresponding hole in the cabinet.

STEP 38:	Again, within the same window, since you are already on concentric, you can continue to concentric mate the remaining hole on the hinge and cabinet together. When you are done you should have the following three constraints.

(NOTE:  An error message may appear depending upon the order you create the constriants.  When this happens users will be unable to select OK, Apply, or even create a new constriant.  At this point users will be required to remove all previous mates used for the two objects and create them again in a different yet logical order.  Example:  If the two centre mates are created first, then mating the two faces together will be an error.  Therefore, touch constrain the two faces first, then the two holes concentrically).

Once you’ve selected the appropriate constraints with the appropriate faces, click Apply and close the assembly constraints window.  Congratulations!  You have just put two components together.  Note that you can also use Preview to verify the constraining of the two components before you apply the constraints.  Click Apply to apply the constraints, then click OK to return to the Add Component window.

constrain three more Hinge A’s to the Cabinet Shell where the Hinge holes are located.  Your assembly should look like that in the figure below.

STEP 39:	Once you have attached the remaining Hinge A’s, you may begin attaching Hinge B’s.

Select Add Existing, and browse for Hinge A (or however you named it).  In the next window verify that you have the following settings:  Reference Set = MODEL, Positioning = By Constraints, Layer Options = Original.  You will be attaching Hinge B’s to Hinge A’s which will only require one constraint instead of three, as when attaching Hinge A’s to the Cabinet Shell.

Select concentric from your assembly constraints window. Now click two different circular edges on Hinge A and B that make the hinges fit together. Note you may have to flip the constraint() if they line up backwards.

Now that one is done, add the other three Hinge B’s to Hinge A’s using the same method.  You may find it is necessary to reposition the hinge component to expose it.  If this is the case, select it from the Assembly Navigator (left uppermost tab), or directly from the modeling window.  When the component is highlighted, right-click and select Move.  Use About an axis and select the centre of any of the cylindrical parts as your rotation axis. If the whole part tries to move when you reposition your hinge you may have to fix constrain then cabinet in place.  A picture is provided above of how the assembly will look like with all the Hinge B’s in place.

STEP 40:	Next you will attach the cabinet doors to the hinges so that the doors open and close the cabinet.  Click on Add Component, and open either the left or right cabinet door.  Verify the following settings:  Reference Set = Model, Positioning = By Constraints, Layer Option = Original.

For the Cabinet Doors, two constraints will be required.  You will use two concentric mates.

You need to concentric mate the outside face of one hole on the cabinet door to it’s corresponding edge on the hinge. Then repeat the process for the other hinge as shown below.

Follow the same instructions to attach the Cabinet Door to the other Hinge B’s (two sets of hinges for one door).  Once that is complete, insert the other Cabinet Door and apply the same mates as before to the Hinge B’s on the other side of the cabinet.

(If you have exited mating mode for some reason before mating the cabinet door with the other hinge, you can still perform the required mate by using the Assembly Constraints() function.  Since the required components of the assembly are already present in your model, you can select the features to mate directly from the model instead of going between the Component Viewer and the main modeling window).

The end product should look similar to the picture below.

STEP 41:	The last and final step is to insert the hinge pins into the Hinge sets where the cylindrical portion of the parts are.  Select Add Component and open the Hinge Pin part with positioning set to By Constraints.

Next use concentric constraint and constrain the bottom top edge of the main body of the hinge pin to the outer inside circular edge of the hinge you would like to insert the pin in as shown below.

Insert the remaining pins in the other hinge sets, as well as the fasteners in the hinge/cabinet holes at the sides using the same constraining process as with the Hinge Pin. Note you may have to reverse some of you mates if the pin sits backwards when you try to insert it in the hinge. Do not forget to save your work.  You have now completed Learning Module #2!

***DON’T FORGET TO CHECK OUT MINI DESIGN PROJECT #2***

- MINI DESIGN PROJECT 2:

ADVANCED BASICS

- Create at least three freehand sketches of the assembly accomplished in Learning Module 2 with possible designs or patterns to decorate the cabinet.  At least one aspect of the design should be a repeated pattern.  Using Moon SandTM, carve out one of the three sketches, carve out the decorated assembly.  Also, sketch at least three possible designs for door handles.

- Select one of your decoration and door handle designs for your cabinet and modify your cabinet model using NX5 to reflect the new design. Use Instance Feature for any repeated patterns.

- Create multiview engineering drawings for each part used in the cabinet assembly (Top, Front, Right, and Isometric views).  No dimensions are required on these engineering drawings for this MDP.

Please submit (in hardcopy):

- Your freehand sketches of at least three designs from Part I.

- The best of your brainstormed designs modeled in NX5.

- Engineering drawings of each part of the assembly that also show your best design.

The figure above shows an example solution to the required design.  You should submit a solution that is different from that shown below.